Integrated Intel Atom D2550 dual core processor(1.86GHz)

Intel ICH10R express chipset

Integrated Intel GMA3600 graphics core, provides VGA display

Supports SO DDR3 1333/1066/800 memory, Max. 4Gb

Provides 1 x PCI slot and MSATA slot

6 x Intel 82583V 1000M Ethernet port, and supports Bypass function

4 x SATA interfaces, and provides 1 50-pin CF card slot

Supports 256 level Watch Dog function

I.D25SL Motherboard Photos

1. Motherboard Front View

2. I/O Panel Side View

3. Memory Side View

II. Motherboard I/O Front View

III. D25SL_B Motherboard Photos

1. Motherboard Front View

2. I/O Panel Side View

3. Memory Side View

| Product Model | D25SL/D25SL_B |
| --- | --- |
| CPU | Onboard Intel Atom D2550 dual core processor(1.86GHz) |
| Chipset | Intel ICH10R express chipset |
| Memory | 1 x SO-DIMM DDR3 memory slot, supports 1066/800 Mhz, Max. 4GB. |
| Video | Integrated Intel GMA3600 graphics core, supports VGA display ouput |
| Expansion Slot | 1 x MSATA slot, supports SSD, USB Wifi 1 x PCI slot |
| LAN | Integrated 6 x Intel 82583V 1000M Ethernet card, support Wake On LAN, PXE booting, among which LAN1-4 supports 2 sets of Bypass function(D25SL cannot support Bypass) |
| Storage | 4 x SATAII HDD interface 3Gb/s 1 x CF card slot |
| Rear I/O Panel | 1 x COM CISCO standard port 2 x USB 2.0 ports 6 x RJ-45 ports 1 x POWER switch 1 x LED1(POWERLED/HDDLED) 1 x 12V DC_JACK input interface(on the side) |
| Internal I/O Interface | 1 x JVGA header, 2 x 5-pin 1 x LAN_LED header, 2 x 13-pin 1 x LPT interface 4 x USB 2.0 headers 1 x RS232 header 1 x PS/2 keyboard and mouse interface 1 x FP1 header(2 x 5-pin, front panel function button and indicating light interface) 1 x ATX_12V, can be used as +12V input and output power supply interface 2 x PWROUT, HDD power supply interface |
| Fan connector | 1 x CPU Fan connector, and 2 x System Fan connectors, 3-pin |
| GPIO | Supports 4 channel GPIO |
| BIOS | AMI 16MB DPI Flash ROM |
| Watch Dog | Integrated in the chip(256 level, 0-255 seconds) |
| PSU | DC_12V single PSU |
| Working Temp | -10℃～60℃ |
| Storing Temp | -20℃～70℃ |
| Working Humidity | 5%-95% relative humidity, non-condensing |
| Dimension | 220mm x 220mm |