Supports Intel LGA771 Xeon series dual core, quad core processors

Intel G41+ICH7R express chipset

Integrated Intel GMA4500 graphics core(supports DirectX 10)

2 x 204-pin dual channel 1333/1066/800 memory, Max. 8Gb

1 x PCIe 8x slot, supports fiber optic module

3 x Serial ATA2 3Gb/s controller

1 x 52P CF card slot

6 x 82574L 1000M auto adaptive Ethernet card

2 x RS232 headers, CISCO standard interface for COM1

G41XE Motherboard Photos

1. Motherboard Front View

2. I/O Panel Front View

3. I/O Panel Side View

4. Memory Side View

| Product Model | FWMG41XE |
| --- | --- |
| CPU | Supports Intel LGA771 Xeon series dual core, quad core processors |
| Chipset | Intel G41+ICH7R Chipset |
| Memory | 2 x SO-DIMM DDR3 memory slot, supports 1333/1066 Mhz, Max. 8GB. |
| Video | Integrated Intel GMA4500 graphics core, supports VGA display ouput |
| Expansion Slot | 1 x PCI-e 8x slot, supports fiber optical and SFP module 1 x PCI slot |
| LAN | Integrated 6 x Intel 82574L 1000M Ethernet chip Supports Wake On LAN, PXE booting, LAN1-4 supports bypass |
| Storage | 3 x SATA2 HDD interfaces, transfer rate reaches 3Gbps, supports RAID function 1 x MSATA slot, supports SSD 1 x IDE interface 1 x CF card slot |
| Rear I/O Panel | 1 x COM CISCO standard port 2 x USB 2.0 ports 6 x RJ-45 ports 1 x POWER switch 1 x LED1(POWERLED/HDDLED) |
| Internal I/O Interface | 1 x JVGA header, 2 x 5-pin 1 x LAN_LED header, 2 x 13-pin 4 x USB 2.0 headers 1 x COM header 1 x FP1 header(2 x 5-pin, front panel function button and indicating light interface) 1 x PS/2 keyboard and mouse interface |
| Fan connector | 1 x CPU Fan connector and 1 x System Fan connector, 3-pin |
| GPIO | Supports 4 channel GPIO |
| BIOS | AMI 8MB DPI Flash ROM |
| Watch Dog | Supports hardware reset function(256 level, 0-255 seconds) |
| PSU | ATX PSU, 24-pin |
| Working Temp | -10℃～60℃ |
| Storing Temp | -20℃～70℃ |
| Working Humidity | 5%-95% relative humidity, non-condensing |
| Dimension | 220mm x 220mm |