Integrated Intel Celeron J1900 quad core processor(2.0GHz)

Intel Bay Trail SoC platform

Built in Intel HD graphics core

1 x 204-pin single channel DDR3 1333/1066 memory, Max. 8Gb

1 x 2.5 inch HDD bay, can install HDD directly onboard, 1 x SATA header, 1 x MSATA slot(support ssd and usb wifi)

4 x Intel 82574L GbE network, 1 set BYPASS

1 x 12V-19V DC_JACK in

Mini ITX Form Factor(170mm x 170mm)

I.FMITX-M9F_B Motherboard Photos

1. Motherboard Front View

2. I/O Panel Front View

3. I/O Panel Side View

4. Memory Side View

| Product Model | FMITX-M9F_B |
| --- | --- |
| CPU | Onboard Intel Celeron J1900 Bay Trail quad core processor(2.0GHz) |
| Chipset | Based on Intel Bay Trail SOC platform, CPU integrated |
| Memory | 1 x SO-DIMM DDR3 memory slot, supports 1333/1066 Mhz, Max. 8GB. |
| Video | Integrated Intel HD graphics core |
| Expansion Slot | 1 x MSATA slot 1 x MINI PCIE slot(supports wifi, 3g module) |
| LAN | Integrated 4 x Intel 82574L 1000M Ethernet card, support Wake On LAN, 1 set BYPASS |
| Storage | 1 x 2.5 inch notebook hdd bay, can install hdd directly 1 x SATAII HDD interface(alternative to 2.5 inch hdd) 1 x MSATA |
| Rear I/O Panel | 1 x COM CISCO standard port 2 x USB ports(Upper USB 2.0, down USB 3.0) 4 x RJ-45 ports 1 x POWER switch 1 x LED1(POWERLED/HDDLED) 1 x 12V-19V DC_JACK input interface |
| Internal I/O Interface | 1 x VGA header, 1 x 12 -pin 1 x LAN_LED header, 2 x 8-pin 4 x USB 2.0 headers 1 x COM header 1 x PS/2 keyboard and mouse interface 1 x FP1 header(2 x 5-pin, front panel function button and indicating light interface) 1 x ATX 4PIN, can be used as 12V-19V input and output power supply interface 1 x 2PIN power header, can be used as 12V-19V input and output power supply interface 1 x HDD power header 1 x SIM slot |
| Fan connector | 1 x CPU Fan connector, and 1 x System Fan connector |
| GPIO | Supports 4 channel GPIO |
| BIOS | AMI 32MB DPI Flash ROM |
| Watch Dog | Integrated in the chip(256 level, 0-255 seconds) |
| PSU | 12V-19V single PSU |
| Working Temp | -10℃～60℃ |
| Storing Temp | -20℃～70℃ |
| Working Humidity | 5%-95% relative humidity, non-condensing |
| Dimension | 170mm x 170mm |