Integrated Intel Celeron 1037U dual core processor(1.8GHz)

Intel NM70 express chipset

Integrated Intel HD graphics core, provides VGA display

Single channel 1333/1066/800 memory, Max. 8Gb

Provides 2 mini PCIe slots, supports SSD/Wifi/3G module

Provides 6 RS232 headers, COM2/3 supports RS232/422/485 modes

6 x Intel 82583V 1000M Ethernet port

Supports SATA, CF card, SSD storage

I. NM70SL Motherboard Photos

1. Motherboard Front View

2. I/O Panel Front View

3. I/O Panel Side View

4. Memory Side View

| Product Model | FMN70SL |
| --- | --- |
| CPU | Onboard Intel Celeron 1037U dual core processor |
| Chipset | Intel NM70 express chipset |
| Memory | 1 x SO-DIMM DDR3 memory slot, supports 1333/1066 Mhz, Max. 8GB. |
| Video | Integrated Intel HD 2500 graphics core, supports VGA display ouput |
| Expansion Slot | 1 x PCI-e 8x slot, supports fiber optical and SFP module, alternative to mini PCIe slot 1 x mini PCIe slot, supports Wifi and 3G module |
| LAN | 6 x Intel 82583V 1000M Ethernet ports |
| Storage | 2 x SATA2 HDD interfaces, transfer rate reaches 3Gbps1 x MSATA slot, 1 x MSATAIII, transit rate reaches 6Gbps 1 x CF card slot |
| Rear I/O Panel | 1 x COM CISCO standard port 2 x USB 2.0 ports 6 x RJ-45 ports 1 x POWER switch 1 x LED1(POWERLED/HDDLED) 1 x 12V DC_JACK |
| Internal I/O Interface | 1 x JVGA header, 2 x 5-pin 1 x LAN_LED header, 2 x 13-pin 1 x LPT interface 4 x USB 2.0 headers 1 x RS232 headers 1 x FP1 header(2 x 5-pin, front panel function button and indicating light interface) 1 x PS/2 keyboard and mouse interface 1 x ATX_4 pin interface 2 x PWROUT, HDD power supply interface |
| Fan connector | 1 x CPU Fan connector, 4-pin 1 x System Fan connector, 3-pin |
| GPIO | Supports 4 channel GPIO |
| BIOS | AMI 16MB DPI Flash ROM |
| Watch Dog | Supports hardware reset function(256 level, 0-255 seconds) |
| PSU | DC_12V single PSU |
| Working Temp | -10℃～60℃ |
| Storing Temp | -20℃～70℃ |
| Working Humidity | 5%-95% relative humidity, non-condensing |
| Dimension | 220 x 220(mm) |