Integrated Intel Atom N2600/N2800 processor(1.6/1.86GHz)

Intel NM10 express chipset

Supports 1 x DDR3 SO-DIMM 2GB, Max. 4Gb(2Gb for N2600)

Supports VGA, HDMI, LVDS sync/async display

Integrated 2 RTL8111E 1000M Ethernet controller

Provides 6 RS232 COM headers, COM2 supports RS232/422/485 mode

2 mini PCIe slot, supports SSD/wireless Wifi

Integrated Realtek ALC662 6 channel high fidelity audio controller

EPIC-N26(28) Motherboard Photos

1. Motherboard Front View

2. Motherboard Back View

3. I/O Panel Front View

4. I/O Panel Side View

5. Memory Side View

| Product Model | EPIC-N26/N28 |
| --- | --- |
| CPU | Integrated Intel Atom dual core N2600 1.66GHz processor Integrated Intel Atom dual core N2800 1.86GHz processor |
| Chipset | Intel NM10 express chipset |
| Memory | 1 x SO-DIMM DDR3 memory slot, supports 1066/800MHz, Max. 2Gb(EPIC-N28 supports 4Gb memory) |
| Video | Integrated Intel GMA3600 Graphics core(EPIC-N28 integrates GMA3650 core), supports VGA/HDMI/LVDS sync/async, LVDS supports single channel 24 bit, supports sync/async |
| Expansion Slot | 1 x mini PCI-e slot, supports wifi |
| LAN | Integrated 2 x RTL8111E 1000M Ethernet controller, supports Wake On Lan, PXE function |
| Storage | 1 x SATA2 HDD interface 1 x MSATA slot, supports SSD |
| Audio | Onboard Realtek ALC662 6 channel high fidelity audio chip, supports MIC, line out |
| Rear I/O Panel | 1 x 12V DC_JACK input 1 x HDMI port 1 x VGA port 2 x USB 2.0 ports 2 x RJ45 LAN ports 1 x Line out port 1 x POWER, 1 x RESET, 1x LED1(POWERLED/HDDLED)(Front panel) |
| Internal I/O Interface | 1 x LVDS connector, 2 x15-pin 1 x JVGA header, same signal with DB15, 2 x 5-pin 6 x RS232 connectors, COM2 supports 485/422 function 1 x TX-RXCOM, RS422/485 mode 4 x USB 2.0 connectors 1 x PS/2 keyboard and mouse interface 1 x AUDIO header, front panel audio interface, 2 x 5-pin 1 x FP1 front panel function button and indicator light header, 2 x 5-pin 1 x PWROUT HDD power supply headers 1 x LVDS INVERTER connector |
| Fan connector | 1 x CPU Fan connector, 3-pin |
| GPIO | Supports 4 channel GPIO |
| BIOS | AMI 16MB Flash ROM |
| Watch Dog | Supports hardware reset function(256 level, 0-255 seconds) |
| PSU | DC_12V single power supply input |
| Working Temp | -10℃～60℃ |
| Storing Temp | -20℃～70℃ |
| Working Humidity | 5%-95% relative humidity, non-condensing |
| Dimension | 148mm x 102mm |