Onboard AMD T56N dual core processor

AMD Hudson M1 chipset

Integrated HD6310 graphics core, supports VGA/HDMI/LVDS sync/async

1 DDR3 2GB express memory, Max. 4G

2 mini PCIe slot, supports SSD/wireless Wifi

Supports 4 x RS232 COM headers, COM2 supports RS232/422/485 mode

Integrated 2 x Intel 82583V 1000M Ethernet controller, supports Wake On LAN

Integrated Realtek ALC662 6 channel high fidelity audio controller

Supports 6 USB 2.0 headers

EPIC-N45 Motherboard Photos

1. Motherboard Front View

2. I/O Panel Front View

3. I/O Panel Side View

4. Memory Side View

| Product Model | EPIC-N45 |
| --- | --- |
| CPU | AMD T56N dual core processor, 1.65GHz |
| Chipset | AMD Hudson M1 chipset |
| Memory | 1 x SO-DIMM DDRIII slot, supports 1066/800 MHz, Max. 4Gb |
| Video | Integrated AMD HD6320 graphics, provides VGA, HDMI, LVDS display output, supports sync/async, 1080P video, LVDS supports dual channel 24 bit |
| Expansion Slot | 1 x mini PCI-e slot, supports wifi/3G module |
| LAN | 2 x RTL8111E 1000M Ethernet controller, supports Wake On Lan, PXE function |
| Storage | 1 x SATA2 HDD interface 1 x JSATAII slot 1 x MSATA slot, supports SSD |
| Audio | Onboard Realtek ALC662 6 channel high fidelity audio chip, supports MIC, line out |
| Rear I/O Panel | 1 x 12V DC_JACK input 1 x HDMI port 1 x VGA port 2 x USB 2.0 ports 2 x RJ45 LAN ports 1 x AUDIO port(Line out) 1 x POWER, 1 x RESET, 1x LED1(POWERLED/HDDLED) |
| Internal I/O Interface | 1 x LVDS connector, 2 x15-pin 1 x JVGA1 header, same signal with DB15, 2 x 5-pin 4 x RS232 connectors, COM1 supports RS232/485/422 function 1 x TX-RXCOM RS422/485 header 4 x USB 2.0 connectors 1 x KB2 header 1 x AUDIO header, front panel audio interface, 2 x 5-pin 1 x FP1 front panel function button and indicator light header, 2 x 5-pin 1 x CIR header 1 x PWROUT HDD power supply headers 1 x LVDS INVERTER connector |
| Fan connector | 1 x CPU Fan connector, 3-pin |
| GPIO | Supports 4 channel GPIO |
| BIOS | AMI 16MB Flash ROM |
| Watch Dog | Supports hardware reset function(256 level, 0-255 seconds) |
| PSU | DC_12V single power supply input |
| Working Temp | -10℃～60℃ |
| Storing Temp | -20℃～70℃ |
| Working Humidity | 5%-95% relative humidity, non-condensing |
| Dimension | 148mm x 102mm |