Integrated Intel Core i3 3217U dual core processor(1.8GHz)

Intel NM70 express chipset

Built in Intel HD graphics core, provides VGA/HDMI/LVDS display output

Onboard DDR3 2GB express memory, Max. 8G

2 mini PCIe slot, supports SSD/wireless Wifi/3G module

Provides 6 RS232 COM headers, COM2 supports RS422/485 mode

Supports 2 Intel 82583V 1000M Ethernet controller, supports Wake On LAN

Integrated Realtek ALC662 6 channel high fidelity audio controller

EPIC-N80_i3 Motherboard Photos

1. Motherboard Front View

3. I/O Panel Front View

| Product Model | EPIC-N80_i3 |
| --- | --- |
| CPU | Integrated Intel Core i3 3217U dual core processor 1.8GHz, TDP 17W |
| Chipset | Intel NM70 express chipset |
| Memory | Onboard DDR3 2GB express memory(4GB, 8GB available) |
| Video | Integrated Intel HD4000 Graphics core, provides VGA/HDMI/LVDS display output, LVDS support dual channel 24 bit, supports sync/async |
| Expansion Slot | 1 x mini PCI-e slot, supports wifi/3G module 1 x MSATA slot, supports SSD, WIFI, transit rate of SSD can reach 6Gb/s |
| LAN | 2 x Intel 82583V 1000M Ethernet controller, supports Wake On Lan |
| Storage | 1 x SATA2 HDD interface, transit rate can reach 3Gb/s 1 x JSATAII slot, supports SATA HDD |
| Audio | Onboard Realtek ALC662 6 channel high fidelity audio chip, supports MIC, line out, power amplifier |
| Rear I/O Panel | 1 x 12V DC_JACK input 1 x HDMI port 1 x VGA port 2 x USB 2.0 ports 2 x RJ45 LAN ports 1 x Line out(green) 1 x MIC(red) 1 x POWER, 1 x RESET, 1x LED1(POWERLED/HDDLED) |
| Internal I/O Interface | 1 x LVDS connector, 2 x15-pin 1 x JVGA1 header, same signal with DB15, 2 x 5-pin 6 x RS232 connectors, COM2 supports 485/422 function 1 x JLPC header, 2 x 6-pin 4 x USB 2.0 connectors 1 x PS/2 keyboard and mouse interface, 1 x 6-pin 1 x AUDIO header, front panel audio interface, 2 x 5-pin 1 x FP1 front panel function button and indicator light header, 2 x 5-pin 1 x SIM slot 2 x JSATA2 headers, 1 x 5-pin 1 x PWROUT HDD power supply headers 1 x LVDS INVERTER connector |
| Fan connector | 1 x CPU Fan connector, 3-pin |
| GPIO | Supports 4 channel GPIO |
| BIOS | AMI 32MB Flash ROM |
| Watch Dog | Supports hardware reset function(256 level, 0-255 seconds) |
| PSU | DC_12V single power supply input |
| Working Temp | -20℃～70℃ |
| Storing Temp | -40℃～80℃ |
| Working Humidity | 5%-95% relative humidity, non-condensing |
| Dimension | 148mm x 102mm |