Embedded Intel Atom D2550 dual core processor(1.86GHz)

Intel Atom D2550 + NM10 express chipset

Built-in Intel GMA3600 series graphics controller(shared memory 128/224Mb)

1 SO-DIMM DDR3 1066/800 RAM, Max. 4Gb

2 x SATA2 3Gb/s controller

Realtek ALC662 6 channel high fidelity audio controller

Integrated 100M or 100M Ethernet card, supports Wake on LAN

Onboard mini PCIe, MSATA slots, supports SSD and Wifi module

Mini ITX Form Factor (170mm x 170mm)

ITX-A25_A2 VER1.2 Motherboard Photos

1. Motherboard Front View

2. I/O Panel Front View

3. I/O Panel Side View

4. Memory Side View

II. ITX-A25_2D6 VER:1.5A Motherboard Photos

1. Motherboard Front View

2. I/O Panel Front View

3. I/O Panel Side View

4. Memory Slot Side View

| Product Model | ITX-A25_A2/ITX-A25_2D6 VER 1.2 |
| --- | --- |
| CPU | Onboard Intel Atom D2550 dual core processor(1.86GHz) |
| Chipset | Intel NM10 Chipset |
| Memory | 1 x SO-DIMM DDR3 memory slot, supports 1066/800 Mhz, Max. 4GB. |
| Graphics | Integrated Intel GMA3600 graphics card, supports VGA/HDMI/LVDS video output (ITX-A25_2D6 LVDS supports dual channel 24 bit) (ITX-A25_A2 LVDS supports single channel 24 bit) |
| Expansion Slot | 1 x PCI slot 1 x mini PCIe slot(supports wifi module) |
| LAN | 2 x RTL8111E 1000M Ethernet controller(ITX-A25_2D6 configures) 1 x RTL8105E 100M Ethernet controller(ITX-A25_A2 configures) |
| Storage | 2 x SATA HDD interfaces(select from SATA2 and SSD, interface exchanged controlled by dial switch) 1 x MSATA slot(supports SSD) |
| Audio | Onboard Realtek ALC662 6 channel high fidelity audio chip, supports MIC, line out. |
| Rear I/O Panel | 1 x RS232 Serial Port, 1/9 pin supports 5V/12V electrifying function 1 x dual layer PS/2 keyboard and mouse connector 4 x USB 2.0 ports 2 x RJ-45 LAN ports(ITX-A25_A2 supports 1 RJ-45) 1 x dual layer standard audio interface(supports MIC, Line out) 1 x LPT port 1 x VGA port |
| Internal I/O Interface | 1 x LVDS connector, 2 x 15-pin 1 x JVGA1 connector, 2 x 5-pin 1 x HDMI connector, 2 x 8-pin 5 x COM connector, COM2 supports 422/485 function(ITX-A25_2D6 configues) 1 x TX-RXCOM 422/485 interface(ITX-A25_2D6 configures) 4 x USB 2.0 connectors 1 x audio connector, front panel audio interface 2 x 5-pin 1 x FP1 front panel function button and indicator light header, 2 x 5-pin 1 x LVDS INVERTER interfacee |
| Fan connector | 1 x CPU Fan connector and 1 x System Fan connector, 3-pin |
| BIOS | AMI 32MB Flash ROM |
| Watch Dog | Supports hardware reset function(256 level, 0-255 seconds) |
| PSU | 20-pin ATX PSU |
| Working Temp | -10℃～60℃ |
| Storing Temp | -20℃～70℃ |
| Working Humidity | 5%-95% relative humidity, non-condensing |
| Dimension | 170mm x 170mm Mini ITX form factor |