Integrated Intel Atom D2550/N2800 processor(1.86GHz)

Intel Atom D2550/D2500 + NM10 express chipset

Built-in Intel GMA3600 series graphics controller(shared memory 128/224Mb)

1 x 204-pin single channel DDR3 1066/800 RAM, Max. 4Gb

2 x SATA2 3Gb/s controller, SATA2 can support SSD

Integrated Realtek ALC662 6 channel high fidelity audio controller

2 x RTL8111E 1000M Ethernet controllers or 1 x RTL8105E 100M Ethernet controller(optional)

6 RS232 Serial ports, 1/9 pin support +5V/+12V switch, supports TTL function

Mini ITX Form Factor (170mm x 170mm)

ITX-H25_25(28) VER1.5 Motherboard Photos

1. Motherboard Front View

2. I/O Panel Front View

3. I/O Panel Side View

4. Memory Side View

II. ITX-H25_2D6 VER:1.5A Motherboard Photos

1. Motherboard Front View

2. I/O Panel Front View

3. I/O Panel Side View

4. Memory Slot Side View

| Product Model | ITX-H25_25(28) ITX-H25_2D6 VER: 1.5A |
| --- | --- |
| CPU | Onboard Intel Atom D2550 dual core processor(1.86GHz) Onboard Intel Atom N2800 dual core processor(1.86GHz)(H25_28) |
| Chipset | Intel NM70 Chipset |
| Memory | 1 x SO-DIMM DDR3 memory slot, supports 1066/800 Mhz, Max. 4GB. |
| Graphics | Integrated Intel GMA3600 graphics card(128/244Mb shared memory) Supports VGA, LVDS video output(LVDS supports single channel 24bit) |
| Expansion Slot | 1 x PCI slot 1 x mini PCIe slot(supports wifi module) |
| LAN | 2 x RTL8111E 1000M Ethernet controller,, supports Wake On Lan, PXE booting. 1 x RTL8105E 100M Ethernet controller |
| Storage | 1 x MSATA slot(supports SSD) 2 x SATA HDD interfaces, select from SATA2 and SSD |
| Audio | Onboard Realtek ALC662 6 channel high fidelity audio chip, supports MIC, line out. |
| Rear I/O Panel | 1 x RS232 Serial Port, 1/9 pin supports 5V/12V electrifying function 1 x 12V DC_JACK input 1 x PS/2 keyboard and mouse connector 4 x USB 2.0 ports 2 x RJ-45 LAN ports(ITX-H25_25(28) supports 1 RJ-45) 1 x dual layer standard audio interface(supports MIC, Line out) 1 x LPT port 1 x VGA port |
| Internal I/O Interface | 1 x LVDS connector, 2 x 10-pin 1 x JVGA1 same signal as DB15, 2 x 5-pin 5 x RS232 connector, pin 1/9 with 5V/12V electrification function 4 x USB 2.0 connectors 1 x audio connector(front panel audio port) 1 x FP1 front panel function button and indicator light header, 2 x 5-pin 1 x ATX_12V interface(2 x 2-pin, supports +12V input and output) 2 x PWROUT, HDD power supply interface 1 x LVDS INVERTER interface |
| Fan connector | 1 x CPU Fan connector and 1 x System Fan connector, 4-pin |
| BIOS | AMI 32MB Flash ROM |
| Watch Dog | Supports hardware reset function(256 level, 0-255 seconds) |
| PSU | DC_12V single power supply |
| Working Temp | -10℃～60℃ |
| Storing Temp | -20℃～70℃ |
| Working Humidity | 0%-95% relative humidity, non-condensing |
| Dimension | 170mm x 170mm Mini ITX form factor |