Supports Intel Atom D425 single core processor(1.80GHz)

Supports Intel Atom D525 dual core processor(1.80GHz)

Intel Atom D525/D425+NM10 express chipset

1 x 204-pin single channel DDR3 1333/1066/800 RAM, Max. 2Gb

1 x PCIe x1 interface, supports WIFI/3G module/SSD, select from Wifi and SSD

1 x LVDS 2 x 10-pin header(supports 24 bit LVDS) optional

Mini ITX Form Factor (170mm x 170mm)

ITX-M58_D56L VER:1.1 Motherboard Photos

1. Motherboard Front View

2. I/O Panel Front View

3. I/O Panel Side View

4. Memory Side View

| Product Model | ITX-M58_D56L VER:1.1 |
| --- | --- |
| CPU | Onboard Intel Atom D525 dual core processor(1.86GHz) |
| Chipset | Intel ICH8M Chipset |
| Memory | 1 x SO-DIMM DDR3 memory slot, supports 1066/800 Mhz, Max. 2GB. |
| Graphics | Integrated Intel GMA3150 graphics card Supports 2 VGA, 1 LVDS(supports single channel 24 bit) video output Supports LVDS/VGA1 sync/async Supports LVDS/VGA2 sync/async Supports VGA1/VGA2 sync/async |
| Expansion Slot | 1 mini PCIe slot, supports Wifi and 3G module 1 PCI slot |
| LAN | 1 x RTL8105E 100M Ethernet controller, supports Wake On Lan, PXE booting. |
| Storage | 1 x MSATA slot(supports SSD) 2 x SATA2 HDD headers, transfer rate reaches 3Gbps |
| Audio | Onboard Realtek ALC662 6 channel high fidelity audio chip, supports MIC, line out. |
| Rear I/O Panel | 1 x 12V DC_JACK input 1 x PS/2 keyboard and mouse connector 1 x LPT port 1 x COM port, pin9 supports 5V/12V electrification function 4 x USB 2.0 ports 1 x RJ-45 LAN ports 1 x audio I/O port(MIC and line-out) |
| Internal I/O Interface | 1 x JVGA1 header, same signal with DB15, 2 x 5-pin 1 x JVGA2 header, 2 x 5-pin 1 x LVDS 2 x 10-pin, supports single 24 bit LVDS port 5 x COM headers, COM2 supports RS422/485 mode 1 x TX-RXCOM RS422/485 mode 1 x audio front panel interface 2 x 5-pin 4 x USB 2.0 connectors 1 x FP1 front panel function button and indicator light header, 2 x 5-pin 1 x PS/2 keyboard mouse header 1 x LVDS power header 1 x 6-pin 2 x PWROUT, HDD power supply interface 1 x ATX_12V(able to function as +12V input and output power interface) 1 x SIM slot |
| Fan connector | 1 x CPU Fan connector and 1 x System Fan connector, 3-pin |
| BIOS | AMI 8MB Flash ROM |
| Watch Dog | Supports hardware reset function(256 level, 0-255 seconds) |
| PSU | DC_12V single power supply |
| Working Temp | -10℃～60℃ |
| Storing Temp | -20℃～70℃ |
| Working Humidity | 0%-95% relative humidity, non-condensing |
| Dimension | 170mm x 170mm Mini ITX form factor |