Supports Intel LGA1155 Socket Core 22nm/32nm i7/i5/i3 CPU

Intel H61 express chipset

Supports 1 x SO-DIMM DDR3 memory slot, Max. 8GB

Provides VGA, DVI port video output, supports synchronization and asynchronization display.

6 RS232 connectors, supports electrification function, COM1 supports RS485/422 modes

Supports 1 x PCI-e 16x slot, 1 x mini PCI-e slot

2 x SATAIII HDD interface, Max. transfer rate 6Gb/s

Integrated Realtek ALC662 6 channel high fidelity audio controller

Mini ITX Form Factor (170x170mm)

ITX-M61 VER:2.2 Motherboard Photos

1. ITX-M61 VER:2.2 Motherboard Front View

2. ITX-M61 VER:2.2 I/O Panel Front View

3. ITX-M61 VER:2.2 I/O Panel Side View

4. ITX-M61 VER:2.2 Memory Side View

| Product Model | ITX-M61 VER:2.2 |
| --- | --- |
| CPU | Supports LGA1155 Intel Core i7/i5/i3 22nm/32nm processor |
| Chipset | Intel H61 Chipset |
| Memory | 1 x SO-DIMM DDR3 memory slot, supports 1066/1333MHz, Max. 8GB |
| Video | Integrated Intel HD graphics controller Supports DVI/VGA synchronization/asynchronization |
| Expansion Slot | 1 x mini PCI-e slot, supports SSD/Wifi/USB Wifi module 1 x PCI-e 2.0 16x slot |
| LAN | 2 x RTL8111E 1000M Ethernet controller, supports Wake On Lan |
| Storage | 2 x SATAII HDD interface, Max. transfer rate 6Gbps |
| Audio | Onboard Realtek ALC662 6 channel high fidelity audio chip, supports MIC, line out. |
| Rear I/O panel | 1 x PS/2 keyboard and mouse connector 4 x USB 2.0 ports 1 x DVI port 1 x VGA port 2 x RS232 ports, COM1 supports RS422/485 modes 2 x RJ-45 LAN ports 1 x dual layer audio port(supports MIC, line out) |
| Internal I/O interface | 4 x RS232 connector(pin-9 supports electrification function) 1 x TX-RXCOM RS422/485 connectors 3 x USB 2.0 connectors 1 x JVGA connector, 2 x 5-pin 1 x FP1(2 x 5-pin front panel function button and indicating light header) |
| Fan connector | 1 x CPU Fan connector 4-pin and 1 x System Fan connector, 3-pin |
| GPIO | Supports 4 channel GPIO |
| BIOS | AMI 32MB Flash ROM |
| Watch Dog | Supports hardware reset function(256 level, 0-255 seconds) |
| PSU | 20-pin ATX power supply |
| Working Temp | -10℃～60℃ |
| Storing Temp | -20℃～70℃ |
| Working Humidity | 5%-95% relative humidity, non-condensing |
| Dimension | 170mm x 170mm Mini ITX form factor |