Intel Atom dual core D2550 processor(1.86GHz)

Intel Atom D2550+NM10 express chipset

Built in Intel GMA3600 graphics core(shared memory of 128/224 Mb)

1 x SO-DIMM DDR3 1066/800 memory, Max. 4Gb

1 x mini-PCIE slot, supports Wifi/3G modules

1 x MSATA(can be connected to SSD, Wifi, select this or SATA2 )

1 RTL8111E 1000M Ethernet controller

Mini ITX form factor(170mm x 170mm)

ITX-M27 Motherboard Photos

1. Motherboard Front View

2. I/O Panel Front View

3. I/O Panel Side View

4. Memory Side View

| Product Model | ITX-M27 |
| --- | --- |
| CPU | Integrated Intel Atom D2550 dual core processor(1.86GHz) |
| Chipset | Intel NM10 Chipset |
| Memory | 1 x SO-DIMM DDR3 memory slot, supports 1066/800 Mhz, Max. 4GB. |
| Video | Built in Intel GMA3600 graphics core(128/224 Mb shared memory), supports 1080P video playback, supports VGA/HDMI/LVDS display output, LVDS, supports single channel 24 bit. |
| Expansion Slot | 1 x mini PCI-e slot, supports wifi/3G module |
| LAN | 1 x RTL8111E 1000M Ethernet controller, supports Wake On Lan |
| Storage | 2 x SATA2 HDD interface, select from SATA2 and SSD, jumper controls switch 1 x MSATA slot, supports SSD and Wifi |
| Audio | Onboard Realtek ALC662 6 channel high fidelity audio chip, supports MIC, line out. |
| Rear I/O Panel | 1 x 12V DC_JACK input 1 x HDMI port 1 x VGA port 4 x USB 2.0 ports 1 x RJ45 LAN port 1 x Line out(green) 1 x MIC(red) |
| Internal I/O Interface | 1 x LVDS connector, 2 x15-pin, single channel 24 bit 1 x JHDMI header, same signal as HDMI, 2 x 8-pin 1 x JVGA header, same signal with DB15, 2 x 5-pin 1 x LPT connector 2 x RS232 connectors 4 x USB 2.0 connectors 1 x KB/MS connector 1 x audio connector(front panel audio port) 1 x CD-IN power amplifier connector, supports direct speaker output 1 x FP1 front panel function button and indicator light header, 2 x 5-pin 1 x LVDS INVERTER connector 1 x ATX_12V power JACK, can be +12v input and output power interface 1 x SIM slot |
| Fan connector | 1 x CPU Fan connector and 1 x System Fan connector, 3-pin |
| GPIO | Supports 4 channel GPIO |
| BIOS | AMI 16MB Flash ROM |
| Watch Dog | Supports hardware reset function(256 level, 0-255 seconds) |
| PSU | DC_12V single power supply input |
| Working Temp | -10℃～60℃ |
| Storing Temp | -20℃～70℃ |
| Working Humidity | 5%-95% relative humidity, non-condensing |
| Dimension | 170mm x 170mm Mini ITX form factor |