Integrated Intel Celeron 1037 dual core processor(1.8GHz)

Intel HM65 express chipset

Integrated Intel HD graphics core, supports VGA, dual HDMI and LVDS video output

SO-DIMM DDR3 memory slot, supports 1333/1066 memory, Max. 8Gb

Onboard 2 mini-PCIE slots, supports SSD/wireless Wifi/USB Wifi/3G modules

Supports 1 2.5’’ HDD bay

Supports 2 RS232 headers

Supports 2 RTL8111E 1000M Ethernet controllers, supports Wake On LAN

Integrated Realtek ALC662 6 channel high definition audio controller

ITX-M11 VER:1.0 Motherboard Photos

1. Motherboard Front View

2. I/O Panel Front View

3. I/O Panel Side View

4. Memory Side View

5. With HDD onboard

| Product Model | ITX-M11 VER: 1.0 |
| --- | --- |
| CPU | Onboard Intel 1037U dual core 22nm processor (1.8Ghz) |
| Chipset | Intel HM65 Chipset |
| Memory | 1 x SO-DIMM DDR3 memory slot, supports 1333/1066 Mhz, Max. 8GB. |
| Video | Integrated Intel HD graphics core Offers 1 VGA, 2 HDMI and 1 LVDS video output(supports dual channel 24bit) Supports HDMI1/VGA sync/async Supports LVDS/VGA sync/async Supports LVDS/HDMI1 sync/async Supports HDMI1/HDMI2 sync/async |
| Expansion Slot | 1 x mini PCI-e slot, supports wifi/3G module 1 x MSATA3 slot, suppports SSD/USB Wifi(by default) |
| LAN | 1 x RTL8105E 100M Ethernet controller, supports Wake On Lan, PXE booting(And can add a Ethernet port through internal header, customization needed) |
| Storage | 2 x SATA3 HDD interface(select from SATA1 and MSATA, SATA1 by default) 1 x SATA3 notebook HDD bay(SATA2 optional, by default notebook HDD) |
| Audio | Onboard Realtek ALC662 6 channel high fidelity audio chip, supports MIC, line out. |
| Rear I/O Panel | 1 x 12V DC_JACK input 1 x HDMI port 1 x VGA port 4 x USB 2.0 ports, 2 of which can support USB 3.0(customization needed) 1 x RJ45 LAN port 1 x Line out(green) 1 x MIC(red) 1 x Power or Reset switch(optional) |
| Internal I/O Interface | 1 x LVDS connector, supports dual channel 24 bit LCD, 2 x15-pin 1 x JHDMI header, 2 x 8-pin 1 x JVGA1 header, same signal with DB15, 12-pin 2 x RS232 connector, pin9 with electrification function 1 x LPT headert 1 x LAN header, 2 x 7-pin 4 x USB 2.0 connectors, if mini PCIe is USB signal, then supports 2 USB 1 x PS/2 mouse and keyboard connector, 6-pin 1 x FP1 front panel function button and indicator light header, 2 x 5-pin 1 x audio connector(front panel audio port) 2 x power amplifier headers 2-pin 1 x SIM slot 1 x PWROUT HDD power supply connector 1 x ATX_12V power JACK, can be +12v input and output power interface 1 x LVDS INVERTER |
| Fan connector | 1 x CPU Fan connector and 1 x System Fan connector, 4-pin |
| GPIO | Supports 4 channel GPIO |
| BIOS | AMI 32MB Flash ROM |
| Watch Dog | Supports hardware reset function(256 level, 0-255 seconds) |
| PSU | ITX-M18_D2 DC_12V single power supply ITX-M18_A2 ATX PSU |
| Working Temp | -10℃～60℃ |
| Storing Temp | -20℃～70℃ |
| Working Humidity | 5%-95% relative humidity, non-condensing |
| Dimension | 170mm x 170mm Mini ITX form factor |