Integrated Intel Celeron J1800 dual core 22nm  processor(2.41GHz), compatible with Intel Bay Trail -I/M/D full series CPU

Intel Bay Trail SOC chipset

Integrated Intel HD graphics core, supports VGA/HDMI video output, supports 1080P

SO-DIMM DDR3 memory slot, supports 1333 memory, Max. 8Gb

Onboard 1 mini-PCIE slots, supports Wifi/3G modules

Supports 1 x 2.5 inch notebook HDD bay

Integrated Realtek ALC662 6 channel high definition audio controller

ITX-M50_N18 VER:1.5 Motherboard Photos

1. Motherboard Front View

2. I/O Panel Front View

3. I/O Panel Side View

4. Memory Side View

| Product Model | ITX-M50 _N18 VER:1.5 |
| --- | --- |
| CPU | Integrated J1800 dual core 22nm processor 2.41GHz |
| Chipset | Intel Bay Trail SOC platform, cpu integrated |
| Memory | 1 x SO-DIMM DDR3 memory slot, supports 1333/1066 Mhz, Max. 8GB. |
| Video | Integrated Intel HD graphics core Offers VGA, HDMI video output |
| Expansion Slot | 1 x mini PCI-e slot, supports wifi/3G module 1 x MSATA slot, suppports SSD |
| LAN | 1 x RTL 1000M Ethernet controller, supports Wake On Lan, PXE booting |
| Storage | 1 x 2.5 inch notebook HDD bay onboard, can install HDD directly 1 x SATA HDD interface(alternative to 2.5 inch HDD) |
| Audio | Onboard Realtek ALC662 6 channel high fidelity audio chip, supports MIC, line out. |
| Rear I/O Panel | 1 x 12-19V DC_JACK input 1 x HDMI port 1 x VGA port 2 x USB 2.0 ports 2 x USB 3.0 ports 1 x RJ45 LAN port 1 x Line out(green) 1 x MIC(red) |
| Internal I/O Interface | 1 x JVGA1 header, same signal with DB15, 12-pin 2 x USB 2.0 headers 1 x USB 3.0 header 1 x FP1 front panel function button and indicator light header, 2 x 5-pin 1 x audio connector(front panel audio port) 1 x power amplifier headers 4pin 1 x PWROUT HDD power supply connector 1 x ATX 4pin power JACK, can be input and output according to power supply output voltage |
| Fan connector | 1 x CPU Fan connector and 1 x System Fan connector |
| GPIO | Supports 4 channel GPIO |
| BIOS | AMI 64MB Flash ROM |
| Watch Dog | Supports hardware reset function(256 level, 0-255 seconds) |
| PSU | DC 12V-19V single power supply input |
| Working Temp | -10℃～60℃ |
| Storing Temp | -20℃～70℃ |
| Working Humidity | 5%-95% relative humidity, non-condensing |
| Dimension | 170mm x 170mm Mini ITX form factor |