FW-7563-2O

1U Rackmount Network Appliance with Intel Core i3/i5/i7 or Xeon with 6 GbE LAN, 2 GbE SFP supporting mSATA and Wi-Fi/LTE card

Overview:

Thank you for choosing the FW-7564-2O. The system is powered by Intel Core Processors, codenamed Ivy Bridge/Sandy Bridge, with the Intel B75 Series Chipset. It supports up to 16GB of DDR3 system memory at 1600MHz on dual-channel DIMM banks. The FW-7564-2O is equipped with advanced I/O capacity which includes an RJ-45 console port, 4 Serial-ATA ports (comply with SATA Standard 3.0 and 2.0), a mini SATA slot and a CompactFlash slot.

The onboard Ethernet ports be configured with bypass depending on the module specification.

Feature:

Adopts the dedicated multi Ethernet application scheme to design

Based on Intel B75 express chipset

Integrated with 6 x Intel 82583V 1000M Ethernet cards, supports 2 pairs BYPASS

Integrated 1 x Intel i350, supporting 2 x GbE SFP ports

Provides 1 Mini PCIe slot, supports Wi-Fi/LTE modules

Provides 1 mSATA slot, supports SSD, transit rate reaches 6Gbps

Supports SSD, SATA HDD, CF card storage

Specification:                            FW-7563-2O

| Model No. |  | FW-7563-2O |
| --- | --- | --- |
| Form Factor |  | 1U Rackmount |
| Platform | Processor Options | Intel Sandy Bridge/ Ivy Bridge 32nm/22nm Core i7/i5/i3, Pentium, Celeron, Xeon E3 1200 v2. |
|  | CPU Socket | LGA1155 |
|  | Chipset | Intel B75 |
| OS Support |  | Linux, Ubuntu, CentOS, pfSense, Windows All |
| BIOS |  | AMI 32M |
| System Memory | Technology | DDR3 1600/1333/1066MHz |
|  | Max. Capacity | 16GB |
|  | Socket | 2 x SO-DIMM |
| Networking | Ethernet Ports | 6 x Intel 82583V GbE LAN 2 x GbE SFP via Intel i350 |
|  | Bypass | 1 pair of BYPASS |
| Storage | HDD Bays | 1 x 3.5 inch, or 2 x 2.5 inch |
|  | CF/SD | 1 x CF socket onboard |
|  | mSATA | 1 x mSATA socket onboard |
| I/O Interface | Reset Button | 1 x Reset Button |
|  | LED | 1 x Power LED, 1 x HDD LED |
|  | Power Button | 1 x Power Button |
|  | Console | 1 x RJ45 Console |
|  | USB | 2 x USB 2.0 |
|  | LCD Module | N/A |
|  | Display | 1 x VGA output |
|  | Power Input | 1 x AC Inlet |
| Expansion | PCIe | N/A |
|  | Mini-PCIe | 1 x Mini PCIe |
| Miscellaneous | Watchdog | Yes |
|  | Internal RTC with Li Battery | Yes |
|  | TPM | Yes |
| Cooling | Processor | 1 x Bronze CPU Fan |
|  | System | 3 x 4020 System Fan |
| Environmental Parameters | Temperature | -15~60℃ |
|  | Humidity | 0% ~ 95%, non condensing |
| Physical Dimensions | (WxHxD) | 430*270*44mm |
|  | Weight | 4.5KG |
| Package Dimensions | (WxHxD) | 530*370*150mm |
|  | Weight | 5.0KG |
| Power | Type/Watts | 220W |
|  | Input | 4A 100-127V~ or 2A 200-240V~ 47-63Hz |
| Approvals and Compliance |  | CE, FCC |

System Structure

Ordering Information

| Part number | PCH | DDR3 | 1Gb LAN port | Bypass | Console | USB 2.0 | 2.5'' HDD | 3.5'' HDD | PCIe Expansion | Power |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FW-7563-2O | B75 | 2 | 6 x Intel 82583V | 2 | 1 | 2 | 2 | 1 | - | AC 220W single ATX |

Packing List

| 1 | SATA data cable |
| --- | --- |
| 2 | Power cord 3P 180cm, China |
| 3 | Rack ears |
| 4 | Accessory screw bag |

Optional Accessories

| 1 | Power cord 3P 180cm, USA |
| --- | --- |
| 2 | Power cord 3P 180cm, Europe |
| 3 | Power cord 3P 180cm, UK |
| 4 | Power cord 3P 180cm, Japan |
| 5 | Power cord 3P 180cm, India |

Contact:

Allen Dai / Sales Manager

Armortec Technology Limited

Email: allen.y.dai@armortec.net

Skype: allen.dai89

Tel: +86 150 1266 9595